Multi-Height and Flow-Regime-Dependent Inputs Promote Day-Ahead Power Forecasting in A Wind Farm

Accurate day-ahead wind farm power forecasting is essential for power‐system operation, electricity markets, and large-scale integration of variable renewable energy (Holttinen, 2005; Mahoney et al., 2012). The development of deep learning models provide substantial advancement for the forecast (Liu & Zhang, 2024). The forecast depends not only on model but also on the physical relevance of meteorological inputs (Optis & Perr-Sauer, 2019). Previous works paid disproportionate attention to models, including model architectures, hyperparameter optimization, and post-processing techniques (Haq et al., 2025; Verdone et al., 2025). In comparison, the importance of atmospheric information is largely overlooked, which leads to a lack of physical foundation in determining the features input into the models.

The wind speed at hub height has been used as the primary feature input into the models (e.g., Mahoney et al., 2012; Ye et al., 2017; Che et al., 2022; Zhang et al., 2025). This practice originates from single-turbine power curve, where power can be characterized by a single hub-height wind speed (Turbines—Part, 2005), and could be supported by the fact that hub-height wind speed shows strong correlation with power (Clifton et al., 2013). However, in wind farms, given that the spatial heterogeneity of wind speed and turbine wake interactions significantly affect the flow field (Wharton & Lundquist, 2012), power forecasting based on only the hub-height speed may not be sufficiently reliable (Buhan et al., 2016). For example, wind speed at 10 m (Chen & Xu, 2022) or other hights (Rubanenko et al., 2020; Chen & Xu, 2022) could be more relevant with power than that at hub-height. The strong correlation between power and non-hub-height wind speeds remains unexplained and highlights the limitations of using purely statistical features in the models. This indicates the necessity of a mechanistic framework explaining when and why wind speeds at different heights could capture the atmospheric forcing relevant to wind-farm power.

A mechanistic understanding begins with free-stream conditions. Single-turbine studies demonstrated that the rotor-equivalent wind speed (REWS)—the swept-area-integrated wind profile—outperforms hub-height wind speed in predicting turbine power (Wagner et al., 2009). Further work under controlled free-stream conditions showed that atmospheric stability can induce 15–20% power variation at the same wind speed, with turbulent kinetic energy identified as a dominant modulation factor (Wharton & Lundquist, 2012). These findings suggest that turbine performance depends on swept-area related wind profile structure, stability, and turbulence, rather than merely on the wind speed at a fixed height such as hub height.

In farm-scale studies, power could also be predicted by the REWS conbined with lapse-rate information (Sasser et al., 2022) better than by the hub-height wind speed. Deep learning studies at the wind farm scale successfully improved power estimation by laveraging multi-height wind measurements compared with hub-height baselines (Sebastiani et al., 2024). These facts highlight the necessity of vertical profile data in power prediction. Nacelle-level averaged winds are more relavant with farm-scale power than hub-height observations (Cutler et al. 2012), indicating the significant spatial heterogeneity of the physical relevance of atmospheric forcing across the farm. In addition, 我们必须再次明确的是运行风电场的环境是包含自由流体和尾流影响流动共同存在的区域，turbine wake interactions and the resultant flow regimes also influence power production. For example, wakes could cause 10%–20% power losses and induce elevated turbulence intensity (Barthelmie et al., 2007; Feroz et al., 2020). Such . More recently, explainable AI studies ranked turbulent variables (i.e., turbulent kinetic energy and turbulence intensity) just behind wind speed in predicting farm-scale power (Optis & Perr-Sauer, 2019; Kim & Kim, 2023). These findings highlight that the effects of model input on power prediction is highly dependent on both vertical wind speed structures and non-speed variables tied to the flow regimes.

Despite the above advancements in power prediction using multi-hight and flow-regime-dependent inputs, fundamental gaps limiting the application of physical insights exist. First, the governing atmospheric timescales remain unclear, hindering the physics of feature selection and failing to explain anomalies such as the high correlation of 10-m wind speed with power. Second, free-stream and wake-flow exhibit distinct dynamics, yet few studies thoroughly compared their importance for power prediction, limiting adaptive feature selection under mixed observational conditions. Third, despite extensive knowledge of wind profile structure, stability, and turbulence, these physical insights have not been translated into day-ahead forecasting strategies. To address these issues in this study, we developed a quantitative framework using full-profile wind observations (10 m, 30 m, 50 m, 70 m) together with power production at a representative wind farm. Our study shows that both multi-height wind profiles and flow-regime characteristics significantly affect day-ahead power forecasting accuracy, which could provide practical guidance for improving NWP-driven day-ahead forecasting.

Barthelmie, R. J., Frandsen, S. T., Nielsen, M. N., Pryor, S. C., Rethore, P. ‐E., & Jørgensen, H. E. (2007). Modelling and measurements of power losses and turbulence intensity in wind turbine wakes at Middelgrunden offshore wind farm. Wind Energy, 10(6), 517–528. https://doi.org/10.1002/we.238

Buhan, S., Ozkazanc, Y., & Cadirci, I. (2016). Wind Pattern Recognition and Reference Wind Mast Data Correlations With NWP for Improved Wind-Electric Power Forecasts. IEEE Transactions on Industrial Informatics, 12(3), 991–1004. https://doi.org/10.1109/TII.2016.2543004

Che, Y., A. Salazar, A., Peng, S., Zheng, J., Chen, Y., & Yuan, L. (2022). A multi-scale model for day-ahead wind speed forecasting: A case study of the Houhoku wind farm, Japan. Sustainable Energy Technologies and Assessments, 52, 101995. https://doi.org/10.1016/j.seta.2022.101995

Chen, Y., & Xu, J. (2022). Solar and wind power data from the Chinese State Grid Renewable Energy Generation Forecasting Competition. Scientific Data, 9(1), 577. https://doi.org/10.1038/s41597-022-01696-6

Clifton, A., Kilcher, L., Lundquist, J. K., & Fleming, P. (2013). Using machine learning to predict wind turbine power output. Environmental Research Letters, 8(2), 024009. https://doi.org/10.1088/1748-9326/8/2/024009

Cutler, N. J., Outhred, H. R., & MacGill, I. F. (2012). Using nacelle‐based wind speed observations to improve power curve modeling for wind power forecasting. Wind Energy, 15(2), 245–258. https://doi.org/10.1002/we.465

Feroz, R. M. A., Javed, A., Syed, A. H., Kazmi, S. A. A., & Uddin, E. (2020). Wind speed and power forecasting of a utility-scale wind farm with inter-farm wake interference and seasonal variation. Sustainable Energy Technologies and Assessments, 42, 100882. https://doi.org/10.1016/j.seta.2020.100882

Haq, I. U., Kumar, A., & Rathore, P. S. (2025). Machine learning approaches for wind power forecasting: a comprehensive review. Discover Applied Sciences, 7(10), 1139. https://doi.org/10.1007/s42452-025-07675-x

Holttinen, H. (2005). Optimal electricity market for wind power. Energy Policy, 33(16), 2052–2063. https://doi.org/10.1016/j.enpol.2004.04.001

Kim, D.-Y., & Kim, B.-S. (2023). Contribution of meteorological factors based on explainable artificial intelligence in predicting wind farm power production using machine learning algorithms. Journal of Renewable and Sustainable Energy, 15(1), 013307. https://doi.org/10.1063/5.0127519

Liu, H., & Zhang, Z. (2024). Development and trending of deep learning methods for wind power predictions. Artificial Intelligence Review, 57(5), 112. https://doi.org/10.1007/s10462-024-10728-z

Mahoney, W. P., Parks, K., Wiener, G., Liu, Y., Myers, W. L., Sun, J., et al. (2012). A Wind Power Forecasting System to Optimize Grid Integration. IEEE Transactions on Sustainable Energy, 3(4), 670–682. https://doi.org/10.1109/TSTE.2012.2201758

Optis, M., & Perr-Sauer, J. (2019). The importance of atmospheric turbulence and stability in machine-learning models of wind farm power production. Renewable and Sustainable Energy Reviews, 112, 27–41. https://doi.org/10.1016/j.rser.2019.05.031

Rubanenko, O., Miroshnyk, O., Shevchenko, S., Yanovych, V., Danylchenko, D., & Rubanenko, O. (2020). Distribution of Wind Power Generation Dependently of Meteorological Factors. In 2020 IEEE KhPI Week on Advanced Technology (KhPIWeek) (pp. 472–477). Kharkiv, Ukraine: IEEE. https://doi.org/10.1109/KhPIWeek51551.2020.9250114

Sasser, C., Yu, M., & Delgado, R. (2022). Improvement of wind power prediction from meteorological characterization with machine learning models. Renewable Energy, 183, 491–501. https://doi.org/10.1016/j.renene.2021.10.034

Sebastiani, A., Angelou, N., & Peña, A. (2024). Wind turbine power curve modelling under wake conditions using measurements from a spinner-mounted lidar. Applied Energy, 364, 122985. https://doi.org/10.1016/j.apenergy.2024.122985

Turbines—Part, W. (2005). 12-1: Power Performance Measurements of Electricity Producing Wind Turbines. British Standard, IEC, 61400–12.

Verdone, A., Panella, M., De Santis, E., & Rizzi, A. (2025). A review of solar and wind energy forecasting: from single-site to multi-site paradigm. Applied Energy, 392, 126016. https://doi.org/10.1016/j.apenergy.2025.126016

Wagner, R., Antoniou, I., Pedersen, S. M., Courtney, M. S., & Jørgensen, H. E. (2009). The influence of the wind speed profile on wind turbine performance measurements. Wind Energy, 12(4), 348–362. https://doi.org/10.1002/we.297

Wharton, S., & Lundquist, J. K. (2012). Atmospheric stability affects wind turbine power collection. Environmental Research Letters, 7(1), 014005. https://doi.org/10.1088/1748-9326/7/1/014005

Ye, L., Zhao, Y., Zeng, C., & Zhang, C. (2017). Short-term wind power prediction based on spatial model. Renewable Energy, 101, 1067–1074. https://doi.org/10.1016/j.renene.2016.09.069

Zhang, Z., Lin, L., Gao, S., Wang, J., Zhao, H., & Yu, H. (2025). A machine learning model for hub-height short-term wind speed prediction. Nature Communications, 16(1), 3195. https://doi.org/10.1038/s41467-025-58456-4

建议题目：Multi-Height and Flow-Regime-Dependent Inputs Enhance Day-Ahead Wind Power Forecasting

深度精修版本329词

Accurate day-ahead wind farm power forecasting is essential for grid operation and the large-scale integration of renewable energy [Ref]. While deep learning models have provided substantial advancements, forecast accuracy depends not only on model architecture but also on the physical relevance of meteorological inputs [Ref]. Previous works have paid disproportionate attention to algorithmic improvements—such as hyperparameter optimization—while largely overlooking the atmospheric physics governing feature selection.

Historically, hub-height wind speed has been the primary input feature, based on the assumption that it sufficiently characterizes power output [Ref]. However, this approach is increasingly challenged by the spatial heterogeneity of wind fields and complex turbine wake interactions [Ref]. For instance, wind speeds at non-hub heights (e.g., 10 m) often exhibit unexplained high correlations with power, highlighting the limitations of purely statistical feature selection. This discrepancy necessitates a mechanistic framework to explain when and why vertical wind structures drive wind farm power.

Mechanistic understanding begins with the distinction between free-stream and wake conditions. Single-turbine studies indicate that the rotor-equivalent wind speed (REWS) outperforms hub-height speed in predicting power [Ref], and atmospheric stability can induce 15–20% power variation via turbulence modulation [Ref]. Crucially, wind farms operate in a mixed environment where free-stream fluids and wake-induced flows coexist. Wake interactions can cause significant power losses and elevated turbulence, meaning that power prediction depends heavily on flow regimes [Ref]. Recent explainable AI studies have further ranked turbulent variables just behind wind speed in importance [Ref].

Despite these advances, fundamental gaps limit the application of physical insights to day-ahead forecasting. First, the governing atmospheric timescales for feature selection remain unclear. Second, the distinct roles of free-stream and wake dynamics in power prediction have rarely been compared adaptively. In this study, we address these issues using full-profile wind observations (10 m, 30 m, 50 m, 70 m) at a representative wind farm. We show that multi-height wind profiles and flow-regime characteristics are critical for reducing forecast errors, providing a physical basis for next-generation NWP-driven forecasting.

温和润色版本414词

Accurate day-ahead wind power forecasting is essential for power-system operation, electricity market trading, and the large-scale integration of variable renewable energy [Refs]. While the rapid development of deep learning models has substantially advanced forecasting capabilities, accuracy depends not only on model architectures but fundamentally on the physical relevance of meteorological inputs [Ref]. Previous research has paid disproportionate attention to algorithmic innovations—such as hyperparameter optimization and post-processing—while often treating feature selection as a black box. Consequently, the atmospheric physics governing wind farm power production remain underrepresented in forecasting strategies.

Historically, wind speed at hub height has been the primary feature input for forecasting models [Refs]. This practice originates from the single-turbine power curve, where power is characterized by a single velocity metric. However, this simplification is increasingly challenged by the complex reality of wind farm environments. Since wind farms operate in a boundary layer marked by significant spatial heterogeneity, power forecasting based solely on hub-height speed lacks reliability [Ref]. Physical studies have demonstrated that the rotor-equivalent wind speed (REWS)—the swept-area-integrated wind profile—outperforms single-point measurements [Ref]. Furthermore, atmospheric stability can induce 15–20% power variation at the same wind speed, with turbulent kinetic energy identified as a dominant modulation factor [Ref].

The complexity is further amplified by flow regimes. Wind farms encompass both free-stream flow and wake-dominated regions. Turbine wake interactions can cause 10–20% power losses and induce elevated turbulence intensity, meaning that the physical drivers of power production fluctuate dynamically with wind direction and atmospheric stability [Refs]. Despite these insights, a gap remains in translating physical knowledge into data-driven forecasting. Most deep learning studies that utilize multi-height data do so without distinguishing between flow regimes. This lack of mechanistic differentiation fails to explain persistent anomalies, such as the unexplained high correlation between near-surface winds (e.g., 10 m) and total power output [Ref].

Here, we bridge the gap between atmospheric flow physics and deep learning forecasting. We propose that the governing atmospheric timescales and vertical structures must be explicitly integrated into the feature selection process. To validate this, we developed a quantitative framework using full-profile wind observations (10 m, 30 m, 50 m, 70 m) combined with power production data at a representative wind farm. We demonstrate that incorporating multi-height wind profiles and flow-regime-dependent inputs—specifically differentiating between free-stream and wake dynamics—significantly promotes day-ahead forecasting accuracy. Our findings provide a mechanistic explanation for the relevance of non-hub-height winds and offer practical guidance for improving Numerical Weather Prediction (NWP)-driven power forecasting.