Introduction

Wind energy has emerged as one of the most rapidly deployed renewable energy sources globally in response to escalating climate challenges and the imperative for energy transition [1,2]. China, in particular, has witnessed remarkable growth in wind power capacity, with cumulative installations reaching 520 GW by the end of 2024, accounting for approximately 50% of global capacity [1]. This expansion forms a cornerstone of efforts to construct a new-type power system capable of accommodating high penetrations of variable renewable generation [3,4]. However, the inherent variability and unpredictability of wind resources pose significant challenges to system operators [5]. Accurate wind power forecasting has thus become essential not only for grid-level applications such as unit commitment and economic dispatch [6], but also for plant-level operations including equipment maintenance scheduling, performance assessment, and participation in electricity markets [7]. Among various forecasting horizons, day-ahead predictions covering lead times from 6 hours to 15 days [8] play a particularly critical role in market clearing and generation planning, directly impacting both economic efficiency and grid stability.

Day-ahead wind power forecasting relies fundamentally on high-quality numerical weather prediction (NWP) outputs [9], which provide meteorological variables as predictive factors to statistical or machine learning models for subsequent error correction and power conversion [10]. This NWP-driven framework has become the dominant paradigm in both research and operational practice [11]. Artificial intelligence methods, including shallow machine learning and deep learning algorithms, have largely supplanted traditional approaches such as theoretical power curve calculations or turbine-level aggregation, owing to their superior ability to capture nonlinear relationships and extract patterns from abundant data [12–14]. However, regardless of model sophistication, the effectiveness of any forecasting system hinges critically on a fundamental question: which meteorological variables should be selected as predictive factors [15–17]? This feature selection problem, essentially the identification of optimal predictors, represents the cornerstone of the entire modeling chain [18]. 缺一句话：又要如何寻找预测因子并更好地建立其和整场发电的代表性关系呢？

我们的切入点/调研回顾将从两个方面展开，因为……To achieve more accurate wind power forecasting, it is essential to consider two interrelated yet relatively independent components: wind speed prediction and power conversion modeling. The former determines the input accuracy to the wind power system, while the latter reflects the efficiency of converting wind energy into electrical power. Together, these components constitute the core of the wind power forecasting framework. This study therefore focuses on identifying physically meaningful predictive factors that link observed winds to power output, bridging the gap between wind prediction and conversion modeling. To clarify the current state of research and identify key challenges, it is essential to systematically review progress in both components.

Given these considerations, research on predictive factors can be broadly divided into two interrelated but distinct branches: wind speed prediction and wind power forecasting. The first branch focuses on accurate prediction of wind speeds that serve as the fundamental input to power models. Research on wind speed prediction for wind energy applications has predominantly focused on hub-height winds, typically at elevations of 70-120 meters where modern wind turbines operate [19]. Recent advances in machine learning have demonstrated remarkable skill in hub-height wind speed forecasting, with models such as CNN-Transformer architectures and multivariate fusion networks achieving substantial improvements over traditional NWP outputs [20,21]. Efforts to enhance prediction accuracy have pursued multiple directions. Global NWP models have been extensively evaluated for their capability to simulate hub-height winds, revealing considerable variability in performance across different systems and regions [22–24]. For example, Jin et al. [25] systematically assessed the forecasting capability of five NWP datasets for hub-height wind speeds and wind ramps at 262 wind farms across China, documenting distinct performance differences among operational models from ECMWF, Meteo-France, Deutscher Wetterdienst, NCEP, and the Canadian Meteorological Centre. Studies examining the Weather Research and Forecasting (WRF) model have documented substantial sensitivity of hub-height wind predictions to the choice of planetary boundary layer parameterization [26], horizontal grid resolution [26], and initial conditions [26,27], with optimal configurations varying by location, season, and atmospheric stability regime [28]. The focus on hub-height characterization extends beyond near-term forecasting to encompass long-term wind resource projections, where climate model outputs are similarly translated to turbine elevations for strategic energy planning [29]. Recent advances in wind resource assessment have increasingly emphasized accurate hub-height wind characterization. To bridge the gap between standard observation levels and turbine altitudes, a variety of estimation techniques have been developed, ranging from traditional power-law extrapolation refined by atmospheric stability corrections to data-driven transfer approaches employing machine-learning models [30–32]. Notably, a subset of research has explored the use of 10-meter surface winds as a proxy when hub-height observations are unavailable [33,34], treating near-surface measurements primarily as a pragmatic alternative rather than investigating their potential intrinsic value for wind-to-power conversion at the farm scale [33].

In contrast, the second branch—wind power forecasting—has evolved along a separate path emphasizing data-driven modeling. Wind power forecasting research has evolved through distinct phases of increasing methodological complexity [11]. Early approaches combined NWP data with manufacturer-provided power curves [35] or individual turbine SCADA data [36] to establish wind-to-power relationships [37]. The advent of data-driven methods marked a paradigm shift, with researchers progressively incorporating more sophisticated preprocessing techniques and model architectures [38]. Contemporary studies exhibit three prominent trends: input complexity [39,40], preprocessing elaboration [41,42], and model advancement [43].

Regarding input predictive factors, the field has transitioned from single-turbine hub-height wind speed to incorporating numerous NWP variables at multiple spatial points and atmospheric levels, with feature sets expanding to include temperature, pressure, humidity, wind direction, and various derived quantities. For instance, Keisler and Le Naour [44] utilized 100-meter wind forecasts at 9-kilometer resolution from ECMWF HRES for regional wind power prediction. Wang et al. [45] employed NWP forecasts at four gradient tower heights (10, 30, 50, and 70 meters) as model inputs, exemplifying the trend toward multi-level meteorological variable integration. Andrade and Bessa [46] incorporated multi-grid-point wind speed components (u and v) along with wind direction from WRF simulations to enhance power forecasting capability, demonstrating the evolution from simple wind speed inputs to comprehensive meteorological fields including historical power data. Lu et al. [47] integrated NWP variables such as wind speed, wind direction, temperature, and cloud cover to achieve short-term power prediction from meteorological predictive factors. These studies illustrate a clear progression in wind power forecasting: from simple hub-height wind speed to multi-level atmospheric profiles, then to diverse meteorological variables, and finally to two-dimensional spatial fields, reflecting increasing input complexity.

Preprocessing pipelines have grown increasingly intricate, employing decomposition methods such as empirical mode decomposition (EMD), ensemble empirical mode decomposition (EEMD), and variational mode decomposition (VMD) to extract multiscale temporal patterns from raw input signals before model ingestion. For example, Ding et al. [48] applied EMD to decompose NWP wind speed into trend and detail components for error correction and power forecasting. Cheng et al. [49] used EEMD to decompose NWP data into multiple intrinsic mode functions (IMFs), applying genetic algorithms for feature selection before inputting to a Transformer-based model for power prediction. Qiao et al. [50] employed improved VMD to process historical power data prior to AI model input for time series forecasting. Zhang et al. [51] utilized complete ensemble empirical mode decomposition with adaptive noise (CEEMDAN) combined with wavelet soft threshold denoising to preprocess wind speed inputs before LSTM-based power prediction. These approaches directly apply diverse signal decomposition techniques to predictive factors—whether NWP outputs or observation-driven data—before model training, representing sophisticated preprocessing strategies.

In fact, the primary research focus and frontier in wind power forecasting have been the exploration and application of model architectures and frameworks. Model architectures have progressed from shallow machine learning algorithms to advanced deep neural networks, including gradient boosting machines (GBM) [52], long short-term memory networks (LSTM) [53], gated recurrent units (GRU) [54], convolutional neural networks (CNN) [55], generative adversarial networks (GAN) [56], graph convolutional networks (GCN) [57], Transformers [58], and various hybrid configurations designed to capture spatiotemporal dependencies [12,13]. A substantial body of work concentrates on this domain, with explosive growth in novel model applications to wind power forecasting tasks. Nevertheless, the community's intense focus on AI model advancement often comes at the expense of physical interpretability and uncertainty understanding [59], resulting in direct input-output modeling workflows focused primarily on algorithmic testing rather than comprehensive system understanding.

该预测因子是能够代表和与整场发电功率建立关系的关键气象因子。

The above literature review reveals a clear divergence between wind speed prediction and power forecasting research. Wind speed prediction emphasizes hub-height wind field characterization and numerical refinement, while the latter prioritizes complex input configurations and algorithmic sophistication. Specifically, both branches implicitly assume hub-height winds as the primary predictor without systematic validation at the wind farm scale. This assumption, inherited from single-turbine power curve theory, may not hold when complex wake interactions and atmospheric boundary layer processes govern farm-level power generation. Yet, no studies have systematically examined whether hub-height winds truly exhibit the strongest relationship with farm power output, or whether winds at other measurement heights might offer superior predictive information through different physical pathways. This disconnect introduces biases in input feature selection, modeling strategies, and uncertainty characterization, fundamentally stemming from insufficient systematic analysis of the wind-to-power conversion process itself and inadequate research on predictor identification. Although recent studies have attempted to enhance model interpretability through feature importance analysis [14], SHAP values [60,61], and embedded methods [62], these efforts largely remain confined to post-hoc analysis of model outputs without revealing the underlying physical mechanisms governing wind energy conversion. The absence of this intermediate process understanding not only limits day-ahead forecasting performance but also hinders the identification of key factors for cross-temporal-scale predictions, including Subseasonal-to-Seasonal and interannual forecasting. Therefore, it is necessary to re-examine the construction and selection of predictive factors within the wind power forecasting framework. By grounding analysis in observational evidence and physical processes, we can reveal stable relationships between winds at different heights and power output, thereby establishing a wind power forecasting framework that integrates physical interpretability with robust consistent predictive behavior.

The identified gaps present both challenges and opportunities for advancing wind power forecasting science. Importantly, nearly all operational wind farms are equipped with gradient observation towers measuring winds at multiple heights and comprehensive power monitoring systems recording farm-level generation [63]. These standard installations provide a universal data foundation for systematic investigation that transcends site-specific limitations. Building upon this ubiquitous observational infrastructure, the present study addresses three fundamental questions that have received insufficient attention: (1) At the wind farm scale, which height exhibits the strongest relationship with power output? (2) What physical mechanisms underlie these relationships? (3) Are these observed patterns sufficiently stable and robust to inform predictive strategies?

The rationale for prioritizing observational analysis before prediction is both scientific and practical [64]. From a scientific perspective, identifying key predictive factors and elucidating attribution mechanisms constitute foundational research that should logically precede forecasting applications. The physical understanding derived from observed wind-to-power relationships can directly guide NWP variable selection, feature engineering design, and prediction strategy optimization, thereby avoiding blind inclusion of all variables or purely trial-and-error data-driven approaches. Moreover, physical comprehension of observed wind-power relationships provides a scientific basis for predictive factor selection across extended temporal scales. Current approaches to Subseasonal-to-Seasonal and interannual forecasting rely predominantly on statistical correlations without physical foundations [65]. The mechanistic insights developed here offer a unified practical framework for predictor identification spanning multiple timescales, from hours to seasons and beyond. From a practical standpoint, this study demonstrates the validity of progressing from observational understanding to predictive implementation through day-ahead forecasting validation.

This study pursues four interconnected objectives that advance understanding of wind-to-power conversion at the wind farm scale:

- Relationship identification – Systematically identify which gradient-height winds exhibit the strongest farm-power relationships;

- Mechanism attribution – Elucidate physical processes governing these relationships through boundary layer dynamics and wake interaction analysis;

- Predictability assessment – Evaluate the forecast skill of identified key factors in operational NWP systems;

- Strategy optimization – Design physics-informed prediction schemes and validate their superiority over conventional approaches.

The research methodology follows a three-stage progression. Stage one conducts comprehensive analysis of observed relationships between gradient tower wind measurements and farm power output, combining statistical diagnostics with physical interpretation to identify dominant predictors and explain their primacy. Stage two assesses the forecast skill of key meteorological variables across multiple NWP systems, determining which models best represent the factors identified as critical in observational analysis. Stage three designs and validates prediction strategies that leverage observational insights, comparing physics-informed approaches against conventional methods to quantify improvements achievable through mechanistic understanding. The remainder of this paper is organized as follows. Section 2 describes the study region, data sources, and analytical methods employed. Section 3 presents results in three parts: observational relationship identification and attribution, NWP predictability assessment, and prediction strategy optimization. Section 4 discusses physical mechanisms in detail, interprets the implications for forecasting practice, and contextualizes findings within the broader literature. Section 5 summarizes principal conclusions and outlines directions for future research.

[1]	Council GWE. GWEC| global wind report 2025. Glob Wind Energy Counc Bruss Belg 2025.

[2]	Marzouk OA. Summary of the 2023 report of TCEP (tracking clean energy progress) by the International Energy Agency (IEA), and proposed process for computing a single aggregate rating. vol. 601, EDP Sciences; 2025, p. 00048.

[3]	Wang Y, Chao Q, Zhao L, Chang R. Assessment of wind and photovoltaic power potential in China. Carbon Neutrality 2022;1:15. https://doi.org/10.1007/s43979-022-00020-w.

[4]	Jiang H, Yao L, Qin J, Bai Y, Brandt M, Lian X, et al. Globally interconnected solar-wind system addresses future electricity demands. Nat Commun 2025;16:4523. https://doi.org/10.1038/s41467-025-59879-9.

[5]	Erdiwansyah, Mahidin, Husin H, Nasaruddin, Zaki M, Muhibbuddin. A critical review of the integration of renewable energy sources with various technologies. Prot Control Mod Power Syst 2021;6:3. https://doi.org/10.1186/s41601-021-00181-3.

[6]	Hetzer J, Yu DC, Bhattarai K. An Economic Dispatch Model Incorporating Wind Power. IEEE Trans Energy Convers 2008;23:603–11. https://doi.org/10.1109/TEC.2007.914171.

[7]	Wang Q, Wu H, Florita AR, Brancucci Martinez-Anido C, Hodge B-M. The value of improved wind power forecasting: Grid flexibility quantification, ramp capability analysis, and impacts of electricity market operation timescales. Appl Energy 2016;184:696–713. https://doi.org/10.1016/j.apenergy.2016.11.016.

[8]	Ruan Z, Wang Y, Lu X, Li D, Qin H, Yu G, et al. A systems-oriented review of China’s wind and solar power development toward carbon neutrality. Technol Rev Carbon Neutrality 2025:9550010. https://doi.org/10.26599/TRCN.2025.9550010.

[9]	Marquis M, Wilczak J, Ahlstrom M, Sharp J, Stern A, Smith JC, et al. Forecasting the Wind to Reach Significant Penetration Levels of Wind Energy. Bull Am Meteorol Soc 2011;92:1159–71. https://doi.org/10.1175/2011BAMS3033.1.

[10]	Jung J, Broadwater RP. Current status and future advances for wind speed and power forecasting. Renew Sustain Energy Rev 2014;31:762–77. https://doi.org/10.1016/j.rser.2013.12.054.

[11]	Costa A, Crespo A, Navarro J, Lizcano G, Madsen H, Feitosa E. A review on the young history of the wind power short-term prediction. Renew Sustain Energy Rev 2008;12:1725–44. https://doi.org/10.1016/j.rser.2007.01.015.

[12]	Yang Y, Lou H, Wu J, Zhang S, Gao S. A survey on wind power forecasting with machine learning approaches. Neural Comput Appl 2024;36:12753–73. https://doi.org/10.1007/s00521-024-09923-4.

[13]	Liu Z, Guo H, Zhang Y, Zuo Z. A Comprehensive Review of Wind Power Prediction Based on Machine Learning: Models, Applications, and Challenges. Energies 2025;18:350. https://doi.org/10.3390/en18020350.

[14]	Dhungana H. A machine learning approach for wind turbine power forecasting for maintenance planning. Energy Inform 2025;8:2. https://doi.org/10.1186/s42162-024-00459-4.

[15]	Yu L, Liu H. Feature Selection for High-Dimensional Data: A Fast Correlation-Based Filter Solution. Proc 20th Int Conf Mach Learn ICML-03 2003:856–63.

[16]	Guyon I, Elisseeff A. An Introduction to Variable and Feature Selection. J Mach Learn Res 2003;3:1157–82. https://doi.org/10.1162/153244303322753616.

[17]	Wojtas MA, Chen K. Feature Importance Ranking for Deep Learning. Proc 34th Int Conf Neural Inf Process Syst 2020;429:5105–14.

[18]	Gyawali PK, Liu X, Zou J, He Z. Ensembling improves stability and power of feature selection for deep learning models. In: Knowles DA, Mostafavi S, Lee S-I, editors. Proc. 17th Mach. Learn. Comput. Biol. Meet., vol. 200, PMLR; 2022, p. 33–45.

[19]	Veers P, Dykes K, Lantz E, Barth S, Bottasso CL, Carlson O, et al. Grand challenges in the science of wind energy. Science 2019;366:eaau2027. https://doi.org/10.1126/science.aau2027.

[20]	Zhang Z, Lin L, Gao S, Wang J, Zhao H, Yu H. A machine learning model for hub-height short-term wind speed prediction. Nat Commun 2025;16:3195. https://doi.org/10.1038/s41467-025-58456-4.

[21]	Hardy L, Finney I. Leveraging state‐of‐the‐art AI models to forecast wind power generation using deep learning. Meteorol Appl 2025;32:e70038. https://doi.org/10.1002/met.70038.

[22]	Djalalova IV, Bianco L, Akish E, Wilczak JM, Olson JB, Kenyon JS, et al. Wind Ramp Events Validation in NWP Forecast Models during the Second Wind Forecast Improvement Project (WFIP2) Using the Ramp Tool and Metric (RT&M). Weather Forecast 2020;35:2407–21. https://doi.org/10.1175/WAF-D-20-0072.1.

[23]	Mo J, Shen Y, Yuan B, Li M, Ding C, Jia B, et al. Assessment of Numerical Forecasts for Hub-Height Wind Resource Parameters during an Episode of Significant Wind Speed Fluctuations. Atmosphere 2024;15:1112. https://doi.org/10.3390/atmos15091112.

[24]	Liu Y, Juliano TW, Krishnamurthy R, Gaudet BJ, Lee J. Linking large-scale weather patterns to observed and modeled turbine hub-height winds offshore of the US West Coast. Wind Energy Sci 2025;10:483–95. https://doi.org/10.5194/wes-10-483-2025.

[25]	Jin C, Yang Y, Han C, Lei T, Li C, Lu B. Evaluation of forecasted wind speed at turbine hub height and wind ramps by five NWP models with observations from 262 wind farms over China. Meteorol Appl 2024;31:e70007. https://doi.org/10.1002/met.70007.

[26]	Siuta D, West G, Stull R. WRF Hub-Height Wind Forecast Sensitivity to PBL Scheme, Grid Length, and Initial Condition Choice in Complex Terrain. Weather Forecast 2017;32:493–509. https://doi.org/10.1175/WAF-D-16-0120.1.

[27]	Yang B, Qian Y, Berg LK, Ma P-L, Wharton S, Bulaevskaya V, et al. Sensitivity of Turbine-Height Wind Speeds to Parameters in Planetary Boundary-Layer and Surface-Layer Schemes in the Weather Research and Forecasting Model. Bound-Layer Meteorol 2017;162:117–42. https://doi.org/10.1007/s10546-016-0185-2.

[28]	Zaman T, Juliano TW, Hawbecker P, Astitha M. On Predicting Offshore Hub Height Wind Speed and Wind Power Density in the Northeast US Coast Using High-Resolution WRF Model Configurations during Anticyclones Coinciding with Wind Drought. Energies 2024;17:2618. https://doi.org/10.3390/en17112618.

[29]	Pryor SC, Coburn JJ, Barthelmie RJ, Shepherd TJ. Projecting Future Energy Production from Operating Wind Farms in North America. Part I: Dynamical Downscaling. J Appl Meteorol Climatol 2023;62:63–80. https://doi.org/10.1175/JAMC-D-22-0044.1.

[30]	Xu C, Hao C, Li L, Han X, Xue F, Sun M, et al. Evaluation of the Power-Law Wind-Speed Extrapolation Method with Atmospheric Stability Classification Methods for Flows over Different Terrain Types. Appl Sci 2018;8:1429. https://doi.org/10.3390/app8091429.

[31]	De Montera L, Berger H, Husson R, Appelghem P, Guerlou L, Fragoso M. High-resolution offshore wind resource assessment at turbine hub height with Sentinel-1 synthetic aperture radar (SAR) data and machine learning. Wind Energy Sci 2022;7:1441–53. https://doi.org/10.5194/wes-7-1441-2022.

[32]	Liu B, Ma X, Guo J, Li H, Jin S, Ma Y, et al. Estimating hub-height wind speed based on a machine learning algorithm: implications for wind energy assessment. Atmospheric Chem Phys 2023;23:3181–93. https://doi.org/10.5194/acp-23-3181-2023.

[33]	Millstein D, Bolinger M, Wiser R. What can surface wind observations tell us about interannual variation in wind energy output? Wind Energy 2022;25:1142–50. https://doi.org/10.1002/we.2717.

[34]	Zhou S, Gao CY, Duan Z, Xi X, Li Y. A robust error correction method for numerical weather prediction wind speed based on Bayesian optimization, variational mode decomposition, principal component analysis, and random forest: VMD-PCA-RF (version 1.0.0). Geosci Model Dev 2023;16:6247–66. https://doi.org/10.5194/gmd-16-6247-2023.

[35]	Wang Y, Hu Q, Srinivasan D, Wang Z. Wind Power Curve Modeling and Wind Power Forecasting With Inconsistent Data. IEEE Trans Sustain Energy 2019;10:16–25. https://doi.org/10.1109/TSTE.2018.2820198.

[36]	Chen N, Qian Z, Nabney IT, Meng X. Wind Power Forecasts Using Gaussian Processes and Numerical Weather Prediction. IEEE Trans Power Syst 2014;29:656–65. https://doi.org/10.1109/TPWRS.2013.2282366.

[37]	Landberg L. Short-term prediction of local wind conditions. J Wind Eng Ind Aerodyn 2001;89:235–45. https://doi.org/10.1016/S0167-6105(00)00079-9.

[38]	Hong T, Pinson P, Fan S, Zareipour H, Troccoli A, Hyndman RJ. Probabilistic energy forecasting: Global Energy Forecasting Competition 2014 and beyond. Int J Forecast 2016;32:896–913. https://doi.org/10.1016/j.ijforecast.2016.02.001.

[39]	Khan S, Mazhar T, Khan MA, Shahzad T, Ahmad W, Bibi A, et al. Comparative analysis of deep neural network architectures for renewable energy forecasting: enhancing accuracy with meteorological and time-based features. Discov Sustain 2024;5:533. https://doi.org/10.1007/s43621-024-00783-5.

[40]	Selvaraj J, Muthuramalingam L, Karthikeyan V, Karthick A, Sathiyaseelan V. Optimizing Wind Energy Integration: A Review of Forecasting Techniques and Emerging Trends. Arch Comput Methods Eng 2025. https://doi.org/10.1007/s11831-025-10442-1.

[41]	Phan Q-T, Wu Y-K, Phan Q-D. An Overview of Data Preprocessing for Short-Term Wind Power Forecasting. 2021 7th Int. Conf. Appl. Syst. Innov. ICASI, Chiayi, Taiwan: IEEE; 2021, p. 121–5. https://doi.org/10.1109/ICASI52993.2021.9568453.

[42]	Zhou Y, Ma L, Ni W, Yu C. Data Enrichment as a Method of Data Preprocessing to Enhance Short-Term Wind Power Forecasting. Energies 2023;16:2094. https://doi.org/10.3390/en16052094.

[43]	Liu H, Zhang Z. Development and trending of deep learning methods for wind power predictions. Artif Intell Rev 2024;57:112. https://doi.org/10.1007/s10462-024-10728-z.

[44]	Keisler J, Le Naour E. WindDragon: automated deep learning for regional wind power forecasting. Environ Data Sci 2025;4:e19. https://doi.org/10.1017/eds.2025.10.

[45]	Wang S, Liu H, Yu G. Short-term wind power combination forecasting method based on wind speed correction of numerical weather prediction. Front Energy Res 2024;12:1391692. https://doi.org/10.3389/fenrg.2024.1391692.

[46]	Andrade JR, Bessa RJ. Improving Renewable Energy Forecasting With a Grid of Numerical Weather Predictions. IEEE Trans Sustain Energy 2017;8:1571–80. https://doi.org/10.1109/TSTE.2017.2694340.

[47]	Lu P, Ye L, Pei M, Zhao Y, Dai B, Li Z. Short-term wind power forecasting based on meteorological feature extraction and optimization strategy. Renew Energy 2022;184:642–61. https://doi.org/10.1016/j.renene.2021.11.072.

[48]	Ding M, Zhou H, Xie H, Wu M, Nakanishi Y, Yokoyama R. A gated recurrent unit neural networks based wind speed error correction model for short-term wind power forecasting. Neurocomputing 2019;365:54–61. https://doi.org/10.1016/j.neucom.2019.07.058.

[49]	Cheng J, Luo X, Jin Z. Integrating domain knowledge into transformer for short-term wind power forecasting. Energy 2024;312:133511. https://doi.org/10.1016/j.energy.2024.133511.

[50]	Qiao B, Liu J, Wu P, Teng Y. Wind power forecasting based on variational mode decomposition and high-order fuzzy cognitive maps. Appl Soft Comput 2022;129:109586. https://doi.org/10.1016/j.asoc.2022.109586.

[51]	Zhang Y, Kong X, Wang J, Wang H, Cheng X. Wind power forecasting system with data enhancement and algorithm improvement. Renew Sustain Energy Rev 2024;196:114349. https://doi.org/10.1016/j.rser.2024.114349.

[52]	Park S, Jung S, Lee J, Hur J. A Short-Term Forecasting of Wind Power Outputs Based on Gradient Boosting Regression Tree Algorithms. Energies 2023;16:1132. https://doi.org/10.3390/en16031132.

[53]	Wang D, Xu M, Guangming Z, Luo F, Gao J, Chen Y. Enhancing wind power forecasting accuracy through LSTM with adaptive wind speed calibration (C-LSTM). Sci Rep 2025;15:5352. https://doi.org/10.1038/s41598-025-89398-y.

[54]	Huang X, Zhang Y, Liu J, Zhang X, Liu S. A Short-Term Wind Power Forecasting Model Based on 3D Convolutional Neural Network–Gated Recurrent Unit. Sustainability 2023;15:14171. https://doi.org/10.3390/su151914171.

[55]	Kazmi S, Gorgulu B, Cevik M, Baydogan MG. A Concurrent CNN-RNN Approach for Multi-Step Wind Power Forecasting 2023. https://doi.org/10.48550/arXiv.2301.00819.

[56]	Liu X, Yu J, Gong L, Liu M, Xiang X. A GCN-based adaptive generative adversarial network model for short-term wind speed scenario prediction. Energy 2024;294:130931. https://doi.org/10.1016/j.energy.2024.130931.

[57]	Qiu H, Shi K, Wang R, Zhang L, Liu X, Cheng X. A novel temporal–spatial graph neural network for wind power forecasting considering blockage effects. Renew Energy 2024;227:120499. https://doi.org/10.1016/j.renene.2024.120499.

[58]	M. R. Sarkar, S. G. Anavatti, T. Dam, M. Pratama, B. A. Kindhi. Enhancing Wind Power Forecast Precision via Multi-head Attention Transformer: An Investigation on Single-step and Multi-step Forecasting. 2023 Int. Jt. Conf. Neural Netw. IJCNN, 2023, p. 1–8. https://doi.org/10.1109/IJCNN54540.2023.10191444.

[59]	Yan J, Möhrlen C, Göçmen T, Kelly M, Wessel A, Giebel G. Uncovering wind power forecasting uncertainty sources and their propagation through the whole modelling chain. Renew Sustain Energy Rev 2022;165:112519. https://doi.org/10.1016/j.rser.2022.112519.

[60]	Liao W, Fang J, Ye L, Bak-Jensen B, Yang Z, Porte-Agel F. Can we trust explainable artificial intelligence in wind power forecasting? Appl Energy 2024;376:124273. https://doi.org/10.1016/j.apenergy.2024.124273.

[61]	Durap A. Explainable deep learning techniques for wind speed forecasting in coastal areas: Integrating model configuration, regularization, early stopping, and SHAP analysis. Neural Comput Appl 2025;37:21219–57. https://doi.org/10.1007/s00521-025-11433-w.

[62]	Yu R, Liu Z, Li X, Lu W, Ma D, Yu M, et al. Scene learning: Deep convolutional networks for wind power prediction by embedding turbines into grid space. Appl Energy 2019;238:249–57. https://doi.org/10.1016/j.apenergy.2019.01.010.

[63]	Turbines—Part W. 12-1: Power Performance Measurements of Electricity Producing Wind Turbines. Br Stand IEC 2005:61400–12.

[64]	Ancell BC, Kashawlic E, Schroeder JL. Evaluation of Wind Forecasts and Observation Impacts from Variational and Ensemble Data Assimilation for Wind Energy Applications. Mon Weather Rev 2015;143:3230–45. https://doi.org/10.1175/MWR-D-15-0001.1.

[65]	Vitart F, Robertson AW. The sub-seasonal to seasonal prediction project (S2S) and the prediction of extreme events. Npj Clim Atmospheric Sci 2018;1:3. https://doi.org/10.1038/s41612-018-0013-0.