Multi-Height and Flow-Regime-Dependent Day-Ahead Power Forecast for Wind Farm

Accurate day-ahead power forecast for wind farm is essential for power‐system operation, electricity markets, and large-scale integration of variable renewable energy (Holttinen, 2005; Mahoney et al., 2012). Deep learning models provide substantial advancement for the forecast (ref.). Forecast depends not only on model but also on the physical relevance of meteorological inputs (Optis & Perr-Sauer, 2019a). Previous works  placed disproportionate emphasis on models, including model architectures, hyperparameter optimization, and post-processing techniques (Haq et al., 2025; Verdone et al., 2025). In comparison, the importance of atmospheric information is largely overlooked, which leads to a lack of physical foundation in determining the features input into the models.

The wind speed at hub height has been used as the primary feature input into the models (e.g., Mahoney et al., 2012; Ye et al., 2017; Che et al., 2022; Zhang et al., 2025). This practice originates from single-turbine power curve, where performance can be characterized by a single inflow height (Turbines—Part, 2005), and could be supported by the fact that hub-height wind speed shows strong correlation with power (Clifton et al., 2013). However, in wind farms, given that the spatial heterogeneity of wind speed and turbine wake interactions significantly affect the flow field (ref.), power forecasting based on only the hub-height speed could be not sufficiently reliable (some ref. for examples). For example, wind speed at 10-m-height could be more relevant with power than that at hub-height (2022). Similar behavior appears in other wind farm observations (Rubanenko et al., 2020; Chen & Xu, 2022). The high dependence of power on wind speed not at hub-height remains unexplained and highlights the limitations of using purely statistical feature in the models. This indicates the necessity of a mechanistic framework explaining when and why wind speed at different heights capture the atmospheric forcing relevant to farm power.

A mechanistic understanding begins with free-flow conditions. Early single-turbine studies demonstrated that the rotor-equivalent wind speed (REWS)—the swept-area–integrated wind profile—outperforms hub-height wind speed in predicting turbine power (Wagner et al., 2009). Further work under controlled free-stream conditions showed that atmospheric stability can induce 15–20% power variation at the same wind speed, with turbulent kinetic energy (TKE) identified as a dominant modulation factor (Wharton & Lundquist, 2012). These findings suggest that turbine performance depends on swept-area related wind profile structure, stability, and turbulence rather than the wind speed at any fixed height.

Studies using operational wind farm observations extended these insights to full-field conditions. Cutler et al. (2012) found nacelle-level averaged winds more representative of farm-wide power than tower-based hub-height observations. Sasser et al. (2022) confirmed the advantages of REWS combined with lapse-rate information. Recent explainable Artificial Intelligence (AI) studies consistently ranked turbulent variables (TKE, TI) directly behind wind speed in predictive importance (Optis & Perr-Sauer, 2019b; Kim & Kim, 2023). Yet these field observations often contain unknown degrees of wake contamination, making it difficult to separate free-stream dynamics from wake-influenced responses.

Recent studies show that using multi-height wind measurements improves power estimation compared with hub-height or REWS-based approaches (Sasser et al., 2022; Sebastiani et al., 2024). These works highlight the benefit of multi-height inputs but do not explain why such improvement occurs or which subset of heights is most effective under different flow regimes, underscoring the need for a mechanistic understanding of wind profile effects.

Wake-induced flow alteration is fundamental in real wind farms. Seminal measurements by Barthelmie et al. (2007) showed that wakes cause 10–20% power losses and elevate turbulence intensity by up to 20%. More recent work (Feroz et al., 2020; Moss et al., 2024) demonstrated strong seasonal and spatial variability of wake, including flow deceleration, local acceleration, and turbulence damping. These findings indicate that effectiveness of features should differ between free-stream and wake-influenced regions and that single-height measurements cannot represent all inflow states.

Synthesizing existing evidence reveals three fundamental gaps. First, the atmospheric timescales governing total-field power remain unclear; without understanding the scale-dependent power response to atmospheric forcing, feature selection lacks physical grounding and cannot explain anomalies such as the 10 m correlation. Second, free-flow and wake-flow regimes exhibit distinct dynamics, yet few studies systematically compare feature importance across these regimes. This limits adaptive feature selection under mixed observational conditions. Third, despite extensive knowledge of wind profile structure, stability, and turbulence, these physical insights have not been translated into operational day-ahead forecasting strategies, which still rely primarily on empirical hub-height inputs.

To address these issues, this study uses full-profile wind observations (10 m, 30 m, 50 m, 70 m) together with whole-farm power production from a representative site and develops a three-tiered framework. First, ensemble empirical mode decomposition (EEMD) and empirical orthogonal function (EOF) analysis are applied to quantify the dominant atmospheric timescales governing farm power and their physical mechanisms. Second, free-stream and wake-flow conditions are classified using wind direction, and SHAP analysis is used to identify differentiated predictor characteristics for each regime. Third, the physically informed predictors identified from observations are integrated with retrospective NWP forecasts (EC-WRF and GFS-WRF) to construct day-ahead power forecasting strategies using LightGBM models tailored to inflow characteristics. In this way, ……

This study makes three key contributions. (1) It establishes that predictor selection should be grounded in atmospheric processes: the EEMD–EOF analyses show that farm power responds primarily to weather-scale forcing expressed through vertical profile changes, which cannot be captured by a single height. (2) It demonstrates that free-stream and wake flows exhibit distinct predictor behaviors: multi-height wind speeds and temperature consistently emerge as core predictors, but optimal height combinations differ across regimes. (3) It provides operational forecasting strategies that combine physically consistent predictors with NWP-driven machine learning models, using reliable wind-direction forecasts from NWP-WRF to enable regime-specific prediction and improve accuracy over hub-height–based baselines.

Barthelmie, R. J., Frandsen, S. T., Nielsen, M. N., Pryor, S. C., Rethore, P. ‐E., & Jørgensen, H. E. (2007). Modelling and measurements of power losses and turbulence intensity in wind turbine wakes at Middelgrunden offshore wind farm. Wind Energy, 10(6), 517–528. https://doi.org/10.1002/we.238

Che, Y., A. Salazar, A., Peng, S., Zheng, J., Chen, Y., & Yuan, L. (2022). A multi-scale model for day-ahead wind speed forecasting: A case study of the Houhoku wind farm, Japan. Sustainable Energy Technologies and Assessments, 52, 101995. https://doi.org/10.1016/j.seta.2022.101995

Chen, Y., & Xu, J. (2022). Solar and wind power data from the Chinese State Grid Renewable Energy Generation Forecasting Competition. Scientific Data, 9(1), 577. https://doi.org/10.1038/s41597-022-01696-6

Clifton, A., Kilcher, L., Lundquist, J. K., & Fleming, P. (2013). Using machine learning to predict wind turbine power output. Environmental Research Letters, 8(2), 024009. https://doi.org/10.1088/1748-9326/8/2/024009

Cutler, N. J., Outhred, H. R., & MacGill, I. F. (2012). Using nacelle‐based wind speed observations to improve power curve modeling for wind power forecasting. Wind Energy, 15(2), 245–258. https://doi.org/10.1002/we.465

Feroz, R. M. A., Javed, A., Syed, A. H., Kazmi, S. A. A., & Uddin, E. (2020). Wind speed and power forecasting of a utility-scale wind farm with inter-farm wake interference and seasonal variation. Sustainable Energy Technologies and Assessments, 42, 100882. https://doi.org/10.1016/j.seta.2020.100882

Haq, I. U., Kumar, A., & Rathore, P. S. (2025). Machine learning approaches for wind power forecasting: a comprehensive review. Discover Applied Sciences, 7(10), 1139. https://doi.org/10.1007/s42452-025-07675-x

Holttinen, H. (2005). Optimal electricity market for wind power. Energy Policy, 33(16), 2052–2063. https://doi.org/10.1016/j.enpol.2004.04.001

Kim, D.-Y., & Kim, B.-S. (2023). Contribution of meteorological factors based on explainable artificial intelligence in predicting wind farm power production using machine learning algorithms. Journal of Renewable and Sustainable Energy, 15(1), 013307. https://doi.org/10.1063/5.0127519

Mahoney, W. P., Parks, K., Wiener, G., Liu, Y., Myers, W. L., Sun, J., et al. (2012). A Wind Power Forecasting System to Optimize Grid Integration. IEEE Transactions on Sustainable Energy, 3(4), 670–682. https://doi.org/10.1109/TSTE.2012.2201758

Moss, C., Maulik, R., & Iungo, G. V. (2024). Augmenting insights from wind turbine data through data-driven approaches. Applied Energy, 376, 124116. https://doi.org/10.1016/j.apenergy.2024.124116

Optis, M., & Perr-Sauer, J. (2019a). The importance of atmospheric turbulence and stability in machine-learning models of wind farm power production. Renewable and Sustainable Energy Reviews, 112, 27–41. https://doi.org/10.1016/j.rser.2019.05.031

Optis, M., & Perr-Sauer, J. (2019b). The importance of atmospheric turbulence and stability in machine-learning models of wind farm power production. Renewable and Sustainable Energy Reviews, 112, 27–41. https://doi.org/10.1016/j.rser.2019.05.031

Rubanenko, O., Miroshnyk, O., Shevchenko, S., Yanovych, V., Danylchenko, D., & Rubanenko, O. (2020). Distribution of Wind Power Generation Dependently of Meteorological Factors. In 2020 IEEE KhPI Week on Advanced Technology (KhPIWeek) (pp. 472–477). Kharkiv, Ukraine: IEEE. https://doi.org/10.1109/KhPIWeek51551.2020.9250114

Sasser, C., Yu, M., & Delgado, R. (2022). Improvement of wind power prediction from meteorological characterization with machine learning models. Renewable Energy, 183, 491–501. https://doi.org/10.1016/j.renene.2021.10.034

Sebastiani, A., Angelou, N., & Peña, A. (2024). Wind turbine power curve modelling under wake conditions using measurements from a spinner-mounted lidar. Applied Energy, 364, 122985. https://doi.org/10.1016/j.apenergy.2024.122985

Turbines—Part, W. (2005). 12-1: Power Performance Measurements of Electricity Producing Wind Turbines. British Standard, IEC, 61400–12.

Verdone, A., Panella, M., De Santis, E., & Rizzi, A. (2025). A review of solar and wind energy forecasting: from single-site to multi-site paradigm. Applied Energy, 392, 126016. https://doi.org/10.1016/j.apenergy.2025.126016

Wagner, R., Antoniou, I., Pedersen, S. M., Courtney, M. S., & Jørgensen, H. E. (2009). The influence of the wind speed profile on wind turbine performance measurements. Wind Energy, 12(4), 348–362. https://doi.org/10.1002/we.297

Wharton, S., & Lundquist, J. K. (2012). Atmospheric stability affects wind turbine power collection. Environmental Research Letters, 7(1), 014005. https://doi.org/10.1088/1748-9326/7/1/014005

Ye, L., Zhao, Y., Zeng, C., & Zhang, C. (2017). Short-term wind power prediction based on spatial model. Renewable Energy, 101, 1067–1074. https://doi.org/10.1016/j.renene.2016.09.069

Zhang, Z., Lin, L., Gao, S., Wang, J., Zhao, H., & Yu, H. (2025). A machine learning model for hub-height short-term wind speed prediction. Nature Communications, 16(1), 3195. https://doi.org/10.1038/s41467-025-58456-4