Multi-Height and Flow-Regime-Dependent Inputs for Day-Ahead Wind Farm Power Forecasting

Accurate day-ahead wind farm power forecasting is essential for power‐system operation, electricity markets, and large-scale integration of variable renewable energy (Holttinen, 2005; Mahoney et al., 2012). Recent deep learning models provide substantial advancement for the forecast (Liu & Zhang, 2024). The forecast depends not only on model but also on the physical relevance of meteorological inputs (Optis & Perr-Sauer, 2019). Previous works placed disproportionate emphasis on models, including model architectures, hyperparameter optimization, and post-processing techniques (Haq et al., 2025; Verdone et al., 2025). In comparison, the importance of atmospheric information is largely overlooked in model-driven studies, which leads to a lack of physical foundation in determining the features input into the models.

The wind speed at hub height has been used as the primary feature input into the models (e.g., Mahoney et al., 2012; Ye et al., 2017; Che et al., 2022; Zhang et al., 2025). This practice originates from single-turbine power curve, where performance can be characterized by a single hub height (Turbines—Part, 2005), and could be supported by the fact that hub-height wind speed shows strong correlation with power (Clifton et al., 2013). However, in wind farms, given that the spatial heterogeneity of wind speed and turbine wake interactions significantly affect the flow field (Wharton & Lundquist, 2012), power forecasting based on only the hub-height speed may not be sufficiently reliable (Buhan et al., 2016). For example, wind speed at 10 m could be more relevant with power than that at hub-height (Chen & Xu, 2022). Similar behavior appears in other wind farm observations (Rubanenko et al., 2020; Chen & Xu, 2022). The strong correlation between power and non-hub-height wind speeds remains unexplained and highlights the limitations of using purely statistical features in the models. This indicates the necessity of a mechanistic framework explaining when and why wind speed at different heights capture the atmospheric forcing relevant to farm power.

A mechanistic understanding begins with free-stream conditions. Early single-turbine studies demonstrated that the rotor-equivalent wind speed (REWS)—the swept-area–integrated wind profile—outperforms hub-height wind speed in predicting turbine power (Wagner et al., 2009). Further work under controlled free-stream conditions showed that atmospheric stability can induce 15–20% power variation at the same wind speed, with turbulent kinetic energy identified as a dominant modulation factor (Wharton & Lundquist, 2012). These findings suggest that turbine performance depends on swept-area related wind profile structure, stability, and turbulence, not merely on the wind speed at a single fixed height such as hub height. Even in farm-wide studies, Sasser et al. (2022) confirmed the advantages of REWS when combined with lapse-rate information, underscoring the necessity of vertical profile data in operational contexts.

Extending these physical insights to operational wind farms introduces significant observational complexity due to turbine wake interactions and the resultant flow regimes. Seminal measurements showed that wakes cause 10–20% power losses and elevate turbulence intensity (Barthelmie et al., 2007; Feroz et al., 2020), creating two distinct, co-existing physical states: free-stream and wake-influenced flow. This spatial heterogeneity means that the physical relevance of atmospheric forcing shifts dramatically across the farm. For instance, Cutler et al. (2012) found nacelle-level averaged winds more representative of farm-wide power than tower-based hub-height observations. Consistent with this, recent explainable AI studies consistently ranked turbulent variables (turbulent kinetic energy and turbulence intensity) directly behind wind speed in predictive importance (Optis & Perr-Sauer, 2019; Kim & Kim, 2023). These findings highlight that the predictive power of inputs is highly dependent on both vertical structure and non-speed variables like turbulence, which are tied to the flow regime.

Motivated by the documented shortcomings of single-height inputs, recent deep learning studies at the wind farm scale have empirically demonstrated the benefit of integrating multi-height wind measurements, leading to improved power estimation compared with hub-height baselines (Sasser et al., 2022; Sebastiani et al., 2024). However, these works lack the mechanistic explanation needed to inform adaptive feature selection. They generally apply multi-height inputs globally, failing to disentangle or exploit the differentiated physical dynamics between the free-stream and wake-flow regimes. This limits their ability to explain why specific non-hub heights capture the atmospheric forcing relevant to farm power.

Synthesizing existing evidence reveals three fundamental gaps that limit the translation of physical insights into operational forecasting strategies. First, the governing atmospheric timescales remain unclear, hindering the physical grounding of feature selection and failing to explain anomalies such as the 10 m correlation. Second, free-stream and wake-flow regimes exhibit distinct dynamics, yet few studies thoroughly compare feature importance across these regimes, limiting adaptive feature selection under mixed observational conditions. Third, despite extensive knowledge of wind profile structure, stability, and turbulence, these physical insights have not been translated into operational day-ahead forecasting strategies, which still rely primarily on empirical hub-height inputs.

To address these issues, this study uses full-profile wind observations (10 m, 30 m, 50 m, 70 m) together with whole-farm power production from a representative site and develops a three-tiered framework. First, ensemble empirical mode decomposition (EEMD) and empirical orthogonal function (EOF) analysis are applied to quantify the dominant atmospheric timescales governing farm power and their physical mechanisms. Second, free-stream and wake-flow conditions are classified using wind direction, and SHAP analysis is used to identify differentiated input characteristics for each regime. Third, the physically informed inputs identified from observations are integrated with retrospective NWP forecasts (EC-WRF and GFS-WRF) to construct day-ahead power forecasting strategies using LightGBM models tailored to inflow characteristics.

This study shows that both multi-height wind profiles and flow-regime characteristics significantly affect day-ahead power forecasting accuracy. It advances the physical understanding of wind farm power generation by demonstrating that vertical wind profiles provide the fundamental atmospheric information required for day-ahead forecasting and by identifying regime-specific height selection. For operational practice, the physically informed forecasting strategies provide actionable guidance for improving NWP-driven day-ahead forecasting.

Barthelmie, R. J., Frandsen, S. T., Nielsen, M. N., Pryor, S. C., Rethore, P. ‐E., & Jørgensen, H. E. (2007). Modelling and measurements of power losses and turbulence intensity in wind turbine wakes at Middelgrunden offshore wind farm. Wind Energy, 10(6), 517–528. https://doi.org/10.1002/we.238

Buhan, S., Ozkazanc, Y., & Cadirci, I. (2016). Wind Pattern Recognition and Reference Wind Mast Data Correlations With NWP for Improved Wind-Electric Power Forecasts. IEEE Transactions on Industrial Informatics, 12(3), 991–1004. https://doi.org/10.1109/TII.2016.2543004

Che, Y., A. Salazar, A., Peng, S., Zheng, J., Chen, Y., & Yuan, L. (2022). A multi-scale model for day-ahead wind speed forecasting: A case study of the Houhoku wind farm, Japan. Sustainable Energy Technologies and Assessments, 52, 101995. https://doi.org/10.1016/j.seta.2022.101995

Chen, Y., & Xu, J. (2022). Solar and wind power data from the Chinese State Grid Renewable Energy Generation Forecasting Competition. Scientific Data, 9(1), 577. https://doi.org/10.1038/s41597-022-01696-6

Clifton, A., Kilcher, L., Lundquist, J. K., & Fleming, P. (2013). Using machine learning to predict wind turbine power output. Environmental Research Letters, 8(2), 024009. https://doi.org/10.1088/1748-9326/8/2/024009

Cutler, N. J., Outhred, H. R., & MacGill, I. F. (2012). Using nacelle‐based wind speed observations to improve power curve modeling for wind power forecasting. Wind Energy, 15(2), 245–258. https://doi.org/10.1002/we.465

Feroz, R. M. A., Javed, A., Syed, A. H., Kazmi, S. A. A., & Uddin, E. (2020). Wind speed and power forecasting of a utility-scale wind farm with inter-farm wake interference and seasonal variation. Sustainable Energy Technologies and Assessments, 42, 100882. https://doi.org/10.1016/j.seta.2020.100882

Haq, I. U., Kumar, A., & Rathore, P. S. (2025). Machine learning approaches for wind power forecasting: a comprehensive review. Discover Applied Sciences, 7(10), 1139. https://doi.org/10.1007/s42452-025-07675-x

Holttinen, H. (2005). Optimal electricity market for wind power. Energy Policy, 33(16), 2052–2063. https://doi.org/10.1016/j.enpol.2004.04.001

Kim, D.-Y., & Kim, B.-S. (2023). Contribution of meteorological factors based on explainable artificial intelligence in predicting wind farm power production using machine learning algorithms. Journal of Renewable and Sustainable Energy, 15(1), 013307. https://doi.org/10.1063/5.0127519

Liu, H., & Zhang, Z. (2024). Development and trending of deep learning methods for wind power predictions. Artificial Intelligence Review, 57(5), 112. https://doi.org/10.1007/s10462-024-10728-z

Mahoney, W. P., Parks, K., Wiener, G., Liu, Y., Myers, W. L., Sun, J., et al. (2012). A Wind Power Forecasting System to Optimize Grid Integration. IEEE Transactions on Sustainable Energy, 3(4), 670–682. https://doi.org/10.1109/TSTE.2012.2201758

Optis, M., & Perr-Sauer, J. (2019). The importance of atmospheric turbulence and stability in machine-learning models of wind farm power production. Renewable and Sustainable Energy Reviews, 112, 27–41. https://doi.org/10.1016/j.rser.2019.05.031

Rubanenko, O., Miroshnyk, O., Shevchenko, S., Yanovych, V., Danylchenko, D., & Rubanenko, O. (2020). Distribution of Wind Power Generation Dependently of Meteorological Factors. In 2020 IEEE KhPI Week on Advanced Technology (KhPIWeek) (pp. 472–477). Kharkiv, Ukraine: IEEE. https://doi.org/10.1109/KhPIWeek51551.2020.9250114

Sasser, C., Yu, M., & Delgado, R. (2022). Improvement of wind power prediction from meteorological characterization with machine learning models. Renewable Energy, 183, 491–501. https://doi.org/10.1016/j.renene.2021.10.034

Sebastiani, A., Angelou, N., & Peña, A. (2024). Wind turbine power curve modelling under wake conditions using measurements from a spinner-mounted lidar. Applied Energy, 364, 122985. https://doi.org/10.1016/j.apenergy.2024.122985

Turbines—Part, W. (2005). 12-1: Power Performance Measurements of Electricity Producing Wind Turbines. British Standard, IEC, 61400–12.

Verdone, A., Panella, M., De Santis, E., & Rizzi, A. (2025). A review of solar and wind energy forecasting: from single-site to multi-site paradigm. Applied Energy, 392, 126016. https://doi.org/10.1016/j.apenergy.2025.126016

Wagner, R., Antoniou, I., Pedersen, S. M., Courtney, M. S., & Jørgensen, H. E. (2009). The influence of the wind speed profile on wind turbine performance measurements. Wind Energy, 12(4), 348–362. https://doi.org/10.1002/we.297

Wharton, S., & Lundquist, J. K. (2012). Atmospheric stability affects wind turbine power collection. Environmental Research Letters, 7(1), 014005. https://doi.org/10.1088/1748-9326/7/1/014005

Ye, L., Zhao, Y., Zeng, C., & Zhang, C. (2017). Short-term wind power prediction based on spatial model. Renewable Energy, 101, 1067–1074. https://doi.org/10.1016/j.renene.2016.09.069

Zhang, Z., Lin, L., Gao, S., Wang, J., Zhao, H., & Yu, H. (2025). A machine learning model for hub-height short-term wind speed prediction. Nature Communications, 16(1), 3195. https://doi.org/10.1038/s41467-025-58456-4