Multi-Height and Flow-Regime-Dependent Inputs Enhance Day-Ahead Wind Power Forecasting

Accurate day-ahead wind power forecasting is essential for power-system operation, electricity market, and the large-scale integration of variable renewable energy (Holttinen, 2005; Mahoney et al., 2012). The forecasting has been substantially advanced by the development of deep learning models (Liu & Zhang, 2024), and depends not only on models but fundamentally on the physical relevance of meteorological inputs (Optis & Perr-Sauer, 2019). Previous research has paid disproportionate attention to the models, including architecture innovation (Bentsen et al., 2023), hyperparameter optimization(Hanifi et al., 2024), and post-processing (Xia et al., 2026). In comparison, input feature selection lacks a mechanistically optimization. Consequently, the atmospheric physics governing wind farm power production remain underrepresented in forecasting.

Traditionally, wind speed at hub height has been the primary feature input into the forecasting models (Mahoney et al., 2012; Ye et al., 2017; Che et al., 2022; Sun & Yang, 2023). This practice originates from the single-turbine power curve, where power can be characterized by a single hub-height wind speed (Turbines—Part, 2005), and could be supported by the strong correlation between the hub-height speed and single turbine power (Clifton et al., 2013). However, this simplification is increasingly challenged by the complex wind farm environments. Since wind farms operate in a boundary layer marked by significant spatial heterogeneity of meterological physics, power forecasting based solely on hub-height wind speed speed lacks reliability (Buhan et al., 2016). Physical studies have demonstrated that the rotor-equivalent wind speed (REWS) (Wagner et al., 2009), which integrates wind profile across the swept area of blades, outperforms single-height measurements in forecasting farm-scale power (Sasser et al., 2022). Moreover, atmospheric stability can induce 15–20% power variation at the same wind speed, with turbulent kinetic energy identified as a dominant modulation factor (Wharton & Lundquist, 2012).

The complexity is further amplified by flow regimes. Wind farms run in the regions encompass both free-stream wake-influeced flows. Turbine wake interactions can cause 10–20% power losses and induce elevated turbulence intensity (Barthelmie et al., 2007; Feroz et al., 2020), meaning that the physical drivers of power production fluctuate dynamically with flow regime and atmospheric stability. More recently, explainable AI studies ranked turbulent variables (i.e., turbulent kinetic energy and turbulence intensity) just behind wind speed in predicting farm-scale power (Optis & Perr-Sauer, 2019; Kim & Kim, 2023). Despite these insights, a gap remains in translating physical knowledge into data-driven forecasting. Most deep learning studies using multi-height data to forecast did not distinguish between flow regimes. This lack of mechanistic differentiation fails to explain persistent anomalies, such as the unexplained high correlation between near-surface winds (e.g., 10-m-height wind speed) and total power output (Rubanenko et al., 2020; Chen & Xu, 2022).

In this study, for bridging the gap between atmospheric flow physics and farm-scale power forecasting, we developed a deep learning-based quantitative framework using wind observations at multi heights (i.e., 10, 30, 50, and 70 m) to forecast the power at a representative wind farm. Our results demonstrate that incorporating multi-height wind profiles and flow-regime-dependent inputs specifically differentiating between free-stream and wake dynamics could significantly promote day-ahead forecasting accuracy. This indicates that the governing atmospheric timescales and vertical structures are necessary to be considered in feature selection for deep learning models. These findings provide a mechanistic explanation for the relevance of non-hub-height winds and offer practical guidance for improving power forecasting driven by Numerical Weather Prediction (NWP).

Barthelmie, R. J., Frandsen, S. T., Nielsen, M. N., Pryor, S. C., Rethore, P. ‐E., & Jørgensen, H. E. (2007). Modelling and measurements of power losses and turbulence intensity in wind turbine wakes at Middelgrunden offshore wind farm. Wind Energy, 10(6), 517–528. https://doi.org/10.1002/we.238

Bentsen, L. Ø., Warakagoda, N. D., Stenbro, R., & Engelstad, P. (2023). Spatio-temporal wind speed forecasting using graph networks and novel Transformer architectures. Applied Energy, 333, 120565. https://doi.org/10.1016/j.apenergy.2022.120565

Buhan, S., Ozkazanc, Y., & Cadirci, I. (2016). Wind Pattern Recognition and Reference Wind Mast Data Correlations With NWP for Improved Wind-Electric Power Forecasts. IEEE Transactions on Industrial Informatics, 12(3), 991–1004. https://doi.org/10.1109/TII.2016.2543004

Che, Y., A. Salazar, A., Peng, S., Zheng, J., Chen, Y., & Yuan, L. (2022). A multi-scale model for day-ahead wind speed forecasting: A case study of the Houhoku wind farm, Japan. Sustainable Energy Technologies and Assessments, 52, 101995. https://doi.org/10.1016/j.seta.2022.101995

Chen, Y., & Xu, J. (2022). Solar and wind power data from the Chinese State Grid Renewable Energy Generation Forecasting Competition. Scientific Data, 9(1), 577. https://doi.org/10.1038/s41597-022-01696-6

Clifton, A., Kilcher, L., Lundquist, J. K., & Fleming, P. (2013). Using machine learning to predict wind turbine power output. Environmental Research Letters, 8(2), 024009. https://doi.org/10.1088/1748-9326/8/2/024009

Feroz, R. M. A., Javed, A., Syed, A. H., Kazmi, S. A. A., & Uddin, E. (2020). Wind speed and power forecasting of a utility-scale wind farm with inter-farm wake interference and seasonal variation. Sustainable Energy Technologies and Assessments, 42, 100882. https://doi.org/10.1016/j.seta.2020.100882

Hanifi, S., Cammarono, A., & Zare-Behtash, H. (2024). Advanced hyperparameter optimization of deep learning models for wind power prediction. Renewable Energy, 221, 119700. https://doi.org/10.1016/j.renene.2023.119700

Holttinen, H. (2005). Optimal electricity market for wind power. Energy Policy, 33(16), 2052–2063. https://doi.org/10.1016/j.enpol.2004.04.001

Kim, D.-Y., & Kim, B.-S. (2023). Contribution of meteorological factors based on explainable artificial intelligence in predicting wind farm power production using machine learning algorithms. Journal of Renewable and Sustainable Energy, 15(1), 013307. https://doi.org/10.1063/5.0127519

Liu, H., & Zhang, Z. (2024). Development and trending of deep learning methods for wind power predictions. Artificial Intelligence Review, 57(5), 112. https://doi.org/10.1007/s10462-024-10728-z

Mahoney, W. P., Parks, K., Wiener, G., Liu, Y., Myers, W. L., Sun, J., et al. (2012). A Wind Power Forecasting System to Optimize Grid Integration. IEEE Transactions on Sustainable Energy, 3(4), 670–682. https://doi.org/10.1109/TSTE.2012.2201758

Optis, M., & Perr-Sauer, J. (2019). The importance of atmospheric turbulence and stability in machine-learning models of wind farm power production. Renewable and Sustainable Energy Reviews, 112, 27–41. https://doi.org/10.1016/j.rser.2019.05.031

Rubanenko, O., Miroshnyk, O., Shevchenko, S., Yanovych, V., Danylchenko, D., & Rubanenko, O. (2020). Distribution of Wind Power Generation Dependently of Meteorological Factors. In 2020 IEEE KhPI Week on Advanced Technology (KhPIWeek) (pp. 472–477). Kharkiv, Ukraine: IEEE. https://doi.org/10.1109/KhPIWeek51551.2020.9250114

Sasser, C., Yu, M., & Delgado, R. (2022). Improvement of wind power prediction from meteorological characterization with machine learning models. Renewable Energy, 183, 491–501. https://doi.org/10.1016/j.renene.2021.10.034

Sun, H., & Yang, H. (2023). Wind farm layout and hub height optimization with a novel wake model. Applied Energy, 348, 121554. https://doi.org/10.1016/j.apenergy.2023.121554

Turbines—Part, W. (2005). 12-1: Power Performance Measurements of Electricity Producing Wind Turbines. British Standard, IEC, 61400–12.

Wagner, R., Antoniou, I., Pedersen, S. M., Courtney, M. S., & Jørgensen, H. E. (2009). The influence of the wind speed profile on wind turbine performance measurements. Wind Energy, 12(4), 348–362. https://doi.org/10.1002/we.297

Wharton, S., & Lundquist, J. K. (2012). Atmospheric stability affects wind turbine power collection. Environmental Research Letters, 7(1), 014005. https://doi.org/10.1088/1748-9326/7/1/014005

Xia, X., Luo, Y., Li, P., Chang, R., Liao, Z., & Huang, L. (2026). Systematic evaluation of transformer-based time series forecasting models for post-processing WRF-simulated wind speed and predicting short-term power output. Applied Energy, 403, 127070. https://doi.org/10.1016/j.apenergy.2025.127070

Ye, L., Zhao, Y., Zeng, C., & Zhang, C. (2017). Short-term wind power prediction based on spatial model. Renewable Energy, 101, 1067–1074. https://doi.org/10.1016/j.renene.2016.09.069