2. Data and Methodology

2.1 Observations and Flow Regime Classification

Observational data were collected from a 201-MW wind farm located in the Hexi Corridor, Gansu, China, characterized by flat terrain and a typical desert Gobi surface. The farm consists of 134 turbines (1.5 MW, hub height 61.5 m, rotor diameter 82 m), with a 70-m met mast situated at the center of the western boundary (see map in Figure X). Due to its strategic location relative to the turbine array, the mast captures distinct flow regimes depending on wind direction. High-resolution vertical wind profiles were measured at four levels (10, 30, 50, and 70 m) with a 15-minute sampling interval from May 2021 to October 2022. Rigorous quality control was applied to remove outliers and fill minor gaps (details in Text S1). To strictly isolate flow dynamics, records were classified into regimes only when wind directions aligned consistently across all four vertical levels (10–70 m) within specific sectors: free-stream (West: 225°–315°) and wake-flow (East: 45°–135°), representing clean inflow and wake-disturbed conditions, respectively (illustrated in Figure X).

2.2 Analytical Framework

To reveal the physical mechanisms governing power production, we employed a multi-dimensional analysis framework (mathematical principles in Text S3). First, we applied Complete Ensemble Empirical Mode Decomposition with Adaptive Noise (CEEMDAN) to decompose wind speed and power series into intrinsic mode functions, identifying the dominant timescales of variability across vertical layers (Text S3.1). Second, Empirical Orthogonal Function (EOF) analysis was performed on the four-level wind profiles to extract the dominant vertical modes of variation, compensating for the lack of direct stability measurements (Text S3.2). Finally, we utilized LightGBM combined with SHAP (SHapley Additive exPlanations) values to quantify feature importance (Text S3.3–S3.4). As validated by Liao et al. (2024), this approach allows us to mechanistically rank the contribution of multi-height winds and temperature inputs under different flow regimes.

2.3 Day-Ahead Forecasting Experimental Design

The forecasting target is day-ahead wind farm power output using Numerical Weather Prediction (NWP) inputs generated by the Weather Research and Forecasting (WRF) model. To account for NWP uncertainty, we utilized retrospective forecasts from two distinct dynamical downscaling configurations driven by the ECMWF-IFS (hereafter EC-WRF) and NCEP-GFS (hereafter GFS-WRF) archives, respectively (physics schemes in Table S2). The NWP fields were spatially and vertically interpolated to the farm location and mast heights (preprocessing in Text S2). To demonstrate the value of regime-dependent inputs, we designed 15 comparative experiments ranging from a single hub-height baseline to our proposed Physics-Informed Fusion Strategy (listed in Table S1). In this proposed strategy, input features are dynamically selected based on the predicted flow regime. Specifically, free-stream conditions utilize 10-m and 70-m wind speeds, whereas wake-affected conditions switch to 10-m and 30-m speeds based on SHAP-identified importance. Predictions from both NWP streams are then integrated via a correction-prediction two-step process. To prevent temporal data leakage, the dataset was split chronologically, with the first 80% used for training and the subsequent 20% reserved strictly for independent testing (implementation details in Text S3.5). Model performance was evaluated using Root Mean Square Error (RMSE) and correlation coefficients, and the statistical significance was verified by the Diebold-Mariano (DM) test (formulas in Text S4).

SI outline

- Text S1. Data Quality Control and Preprocessing

- Text S2. Operational NWP Configuration and Temporal Scheduling

- Text S3. Methodological Principles

- Text S4. Evaluation Metrics and Statistical Significance

- Table S1. Summary of Experimental Strategies

- Table S2. Detailed configuration of the WRF model domains and physics parameterization schemes

- Table S3. Hyperparameter Configuration for LightGBM Models

- Figure S1. Observational setup and wind farm layout

- References

Text S1. Data Quality Control and Preprocessing

To ensure data integrity, rigorous quality control (QC) procedures were applied to the raw 15-minute observational datasets. The QC process involved three sequential steps: (1) Missing Value Removal: Any temporal record containing missing values (NaN) in either the meteorological mast data or the wind farm power logs was entirely discarded. (2) Anomaly Detection: We identified and removed "frozen" sensor data, defined as records where wind speed or power readings remained identical for three or more consecutive timestamps. (3) Gap Filling: Isolated missing values (single timestamps) were filled using linear interpolation from the immediately preceding and succeeding valid records. However, continuous gaps exceeding 30 minutes were left as missing to avoid introducing artificial trends. After QC, the dataset comprised 52,318 valid samples, ensuring statistical robustness for model training and evaluation.

Text S2. Operational NWP Configuration and Temporal Scheduling

To ensure the practical applicability of the forecasting strategies, the NWP data generation strictly adhered to the operational standards of the local grid dispatching center in Gansu. The day-ahead forecasting window targets the 24-hour period of the following day (Day+1, 00:00–23:45 BJT). The WRF model is initialized daily at 20:00 BJT (Beijing Time), utilizing the latest available global forcing data. A critical 28-hour operational latency exists between the model initialization and the start of the target forecast window, accounting for data transmission, computation, and the strict grid reporting deadline. Consequently, for each daily cycle, the first 28 hours are treated as a spin-up period and discarded. The subsequent 0–24 hour segment is extracted to evaluate day-ahead performance.

Regarding the model architecture, the WRF model (Version 4.2) was configured with nested domains to dynamically downscale the global forcing data. The GFS-driven configuration employed a three-domain nesting strategy with horizontal resolutions of 27, 9, and 3 km, whereas the EC-driven configuration adopted a two-domain strategy (9 and 3 km). Both configurations utilized 51 terrain-following vertical levels. Physics parameterizations included the Morrison 2-moment microphysics, RRTMG radiation, Noah Land Surface Model, and the Yonsei University (YSU) planetary boundary layer scheme (details in Table S2). This configuration follows the operational system systematically evaluated in Xia et al. (2026), ensuring computational stability and consistency across the 3-year hindcast period.

Text S3. Methodological Principles

Text S3.1 CEEMDAN for Timescale Decomposition

To decouple the multiscale characteristics of wind speed and power time series, we employed Complete Ensemble Empirical Mode Decomposition with Adaptive Noise (CEEMDAN) (Torres et al., 2011). This method improves upon standard EEMD (Wu & Huang, 2009) by adding adaptive white noise at each decomposition stage to resolve mode-mixing issues. The decomposition process represents the original time series  as a superposition of Intrinsic Mode Functions (IMFs) and a residual trend:

|  |  | Equation (1) |
| --- | --- | --- |

Where  denotes the original meteorological or power time series,  represents the -th intrinsic mode function corresponding to a specific frequency band (ranging from high-frequency turbulence to synoptic-scale variations),  is the total number of modes, and  is the monotonic residue representing the long-term trend. In this study, this decomposition allows us to isolate physically distinct timescales and analyze their respective correlations with power generation.

Text S3.2 Empirical Orthogonal Function (EOF) Analysis

To characterize the vertical structure of the wind profile without direct turbulence measurements, we applied EOF analysis (Wilks, 2011) to the wind speed vectors measured at four heights (10, 30, 50, and 70 m). EOF analysis decomposes the spatiotemporal wind field into orthogonal spatial modes and temporal coefficients. The wind speed vector field  is approximated by:

|  |  | Equation (2) |
| --- | --- | --- |

Where  represents the wind speed anomalies at height  and time .  is the spatial eigenvector (loading) of the -th mode, describing the vertical distribution pattern (e.g., uniform flow or shear).  is the principal component time series representing the temporal evolution of the -th spatial mode.  is the number of retained modes. Typically, the first mode (EOF1) captures the bulk wind speed energy, while the second mode (EOF2) serves as a proxy for vertical shear and atmospheric stability.

Text S3.3 LightGBM Algorithm

We utilized Light Gradient Boosting Machine (LightGBM) (Ke et al., 2017) as the core regression framework. LightGBM is an ensemble learning algorithm based on decision trees, designed to approximate the complex, non-linear function mapping meteorological inputs to power output. The final prediction is obtained by summing the outputs of a sequence of decision trees:

|  |  | Equation (3) |
| --- | --- | --- |

Where represents the predicted wind power,  is the total number of trees (iterations), and  denotes the function learned by the -th decision tree given the input feature vector  (e.g., wind speeds, direction, temperature). The algorithm minimizes the objective function by iteratively adding trees that fit the residual errors of the previous steps, making it highly effective for correcting systematic biases in NWP data.

Text S3.4 SHapley Additive exPlanations (SHAP)

To interpret the "black-box" nature of the LightGBM model and mechanistically guide our feature selection, we employed the SHAP framework (Lundberg & Lee, 2017). Based on cooperative game theory, SHAP decomposes the model's prediction into the additive contributions of each feature. The explanation model is defined as:

|  |  | Equation (4) |
| --- | --- | --- |

Where  is the model's predicted output for a specific instance,  is the base value (the average model output over the training dataset),  is the number of input features, and  is the SHAP value for the -th feature. A positive   indicates that feature  increases the power output relative to the baseline, while a negative value indicates a decrease. Following Liao et al. (2024), we aggregated the absolute SHAP values  to rank the importance of vertical wind layers under different flow regimes, providing the physical justification for our regime-dependent input strategy.

Text S3.5 Model Implementation and Hyperparameter Settings

Input Feature Configuration: The input feature vector () was dynamically tailored to the specific experimental phase.

Diagnostic Phase (Observation-based): To maximize explanatory power for SHAP analysis, the input vector included raw observational variables alongside physics-informed engineering features—specifically wind speed cubics () to mimic the power curve, and temporal lags (1 h, 24 h) to capture system inertia.

Forecasting Phase (NWP-based): To ensure operational robustness and strictly prevent look-ahead bias in day-ahead scenarios, the input vector was restricted to raw meteorological variables (interpolated NWP wind speeds, direction, and temperature) without autoregressive lag terms. The model relied on its inherent non-linear mapping capability to capture power curve relationships directly from these inputs.

Data Splitting Strategy: To prevent temporal data leakage, the dataset was split chronologically, with the first 80% used for model training and validation, and the subsequent 20% reserved strictly for independent testing.

Hyperparameter Optimization: To ensure robust generalization and prevent overfitting, the LightGBM models (both for wind correction and power prediction) were trained using a Gradient Boosting Decision Tree (GBDT) backbone with an Early Stopping strategy. Instead of performing an exhaustive grid search for every fold, we employed a standardized set of hyperparameters that balance model complexity and training efficiency. Crucially, the number of boosting iterations (trees) was dynamically determined by monitoring the RMSE on the validation set. The training process automatically stopped if the validation error did not decrease for 10 consecutive rounds (stopping_rounds=10). This regularization technique ensures that the model captures the underlying physical patterns without memorizing noise. The specific hyperparameter configuration used in this study is detailed in Table S3.

Text S4. Evaluation Metrics and Statistical Significance

Text S4.1 Performance Metrics

To quantitatively evaluate the forecasting accuracy, we employed two standard metrics: Root Mean Square Error (RMSE) and the Spearman Rank Correlation Coefficient ().

The RMSE measures the average magnitude of the forecast errors:

|  |  | Equation (X) |
| --- | --- | --- |

Given the inherent non-linear nature of wind power generation characteristics, we selected the Spearman Rank Correlation Coefficient () rather than the standard Pearson correlation. Spearman correlation assesses the monotonic relationship between predicted and observed sequences based on their ranks, offering greater robustness against outliers and non-linear dependencies:

|  |  | Equation (X) |
| --- | --- | --- |

Where  is the number of test samples, and  is the difference between the ranks of the observed and predicted power values at time .

Text S4.2 Diebold-Mariano (DM) Test

To verify whether the improvement of the proposed strategy over the baseline is statistically significant, we applied the Diebold-Mariano (DM) test [Ref: Diebold & Mariano, 1995]. The test compares the forecast accuracy of two models based on their loss differentials.

Let  and  be the forecast errors of the baseline model and the proposed model, respectively. The loss differential sequence  is defined using the squared error loss function:

|  |  | Equation (X) |
| --- | --- | --- |

The DM test statistic is calculated as:

|  |  | Equation (X) |
| --- | --- | --- |

Where  is the sample mean of the loss differential, and  is the asymptotic variance of . Under the null hypothesis  (i.e., both models have equal predictive accuracy), the DM statistic follows a standard normal distribution . A positive DM value indicates that the proposed model has a smaller error than the baseline. In this study, we consider the improvement significant if the -value is less than 0.05 (confidence level of 95%).

Table S1. Summary of Experimental Strategies

Table S1. Detailed configuration of the 18 forecasting experimental strategies.

|  | Strategy Name | NWP Source | Input Features (Heights) | Processing Flow |
| --- | --- | --- | --- | --- |
| Baseline | G-70-P | GFS-WRF | 70 m (Hub-height) | End-to-End Power Prediction |
| Single-Level | G-10-P | GFS-WRF | 10 m | End-to-End Power Prediction |
|  | G-70-WP | GFS-WRF | 70 m | Wind Correction to Power |
|  | G-10-WP | GFS-WRF | 10 m | Wind Correction to Power |
|  | E-70-P | EC-WRF | 70 m | End-to-End Power Prediction |
|  | E-10-P | EC-WRF | 10 m | End-to-End Power Prediction |
|  | E-70-WP | EC-WRF | 70 m | Wind Correction to Power |
|  | E-10-WP | EC-WRF | 10 m | Wind Correction to Power |
|  | EG-70-WP | Both | 70 m (Dual-source) | Wind Correction to Power |
|  | EG-10-WP | Both | 10 m (Dual-source) | Wind Correction to Power |
| Multi-Level | G-B-WP | GFS-WRF | 10 m, 70 m | Profile Correction to Power |
|  | E-B-WP | EC-WRF | 10 m, 70 m | Profile Correction to Power |
|  | EG-B-WP | Both | 10 m, 70 m (All sources) | Profile Correction to Power |
|  | Fusion-B-WP | Both | All levels, Temperature | Profile Correction to Power |
| Proposed | Fusion-Director-WP | Both | Regime-Dependent Selection* | Regime-Aware Profile Correction to Power |

Note: For the proposed "Fusion-Director-WP" strategy, inputs are dynamically selected based on flow regimes: 10 m & 70 m wind speeds for free-stream sectors, and 10 m & 30 m wind speeds for wake-affected sectors. For all dual-source strategies (GFS & EC), the corrected wind estimates from both members are combined via equal-weighted averaging before power conversion.

Table S2. Detailed configuration of the WRF model domains and physics parameterization schemes

| Configuration Type | Setting / Scheme |
| --- | --- |
| Horizontal Resolution |  |
| GFS-Driven Configuration | Domain 1 (27 km), Domain 2 (9 km), Domain 3 (3 km) |
| EC-Driven Configuration | Domain 1 (9 km), Domain 2 (3 km) |
| Vertical Resolution | 51 levels (terrain-following) |
| Physics Parameterizations |  |
| Microphysics | Morrison 2-moment scheme |
| Longwave Radiation | RRTMG scheme |
| Shortwave Radiation | RRTMG scheme |
| Planetary Boundary Layer | Yonsei University (YSU) scheme |
| Surface Layer | MM5 similarity |
| Land Surface | Unified Noah Land Surface Model |
| Cumulus Parameterization* | Kain-Fritsch scheme |

Note: The Cumulus Parameterization scheme is activated only for domains with resolutions > 4 km (i.e., 27 km and 9 km grids). It is turned off for the 3 km domain to allow for explicit convection.

Table S3. Hyperparameter Configuration for LightGBM Models

| Hyperparameter | Value | Description |
| --- | --- | --- |
| Boosting Type | GBDT | Traditional Gradient Boosting Decision Tree |
| Objective | Regression (L2) | Minimizes the squared error loss |
| Learning Rate | 0.1 | Step size shrinkage used in updates |
| Num Leaves | 31 | Maximum tree leaves for base learners |
| Feature Fraction | 0.9 | Randomly select 90% of features per iteration |
| Max Boosting Rounds | 100 | Maximum limit for iterations |
| Early Stopping | 10 rounds | Stops if validation RMSE implies no improvement |

Note: This configuration was applied consistently across all experimental strategies to ensure fair comparison.

Figure S1. Observational setup and wind farm layout. (a) Photographic view of the 70-m meteorological mast equipped with anemometers and wind vanes at four vertical levels (10, 30, 50, and 70 m), and a temperature sensor at 10 m. (b) Schematic diagram of the wind farm layout illustrating the relative position of the mast (red triangle/structure on the left) to the turbine array. The mast is located 586 m west of the first turbine row, establishing a clear distinction between free-stream inflow (Westerly winds) and wake-affected flow (Easterly winds). The embedded wind rose displays the prevailing wind directions at the site.

References

Ke, G., Meng, Q., Finley, T., Wang, T., Chen, W., Ma, W., et al. (2017). Lightgbm: a highly efficient gradient boosting decision tree. Advances in Neural Information Processing Systems, 30.

Liao, W., Fang, J., Ye, L., Bak-Jensen, B., Yang, Z., & Porte-Agel, F. (2024). Can we trust explainable artificial intelligence in wind power forecasting? Applied Energy, 376, 124273. https://doi.org/10.1016/j.apenergy.2024.124273

Lundberg, S. M., & Lee, S.-I. (2017). A unified approach to interpreting model predictions. Advances in Neural Information Processing Systems, 30.

Torres, M. E., Colominas, M. A., Schlotthauer, G., & Flandrin, P. (2011). A complete ensemble empirical mode decomposition with adaptive noise. In 2011 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP) (pp. 4144–4147). Prague, Czech Republic: IEEE. https://doi.org/10.1109/ICASSP.2011.5947265

Wilks, D. S. (2011). Statistical methods in the atmospheric sciences (Vol. 100). Academic press.

Wu, Z., & Huang, N. E. (2009). Ensemble empirical mode decomposition: a noise-assisted data analysis method. Advances in Adaptive Data Analysis, 1(1), 1–41. https://doi.org/10.1142/S1793536909000047